PERANCANGAN SISTEM DIGITAL

NAME: MUHAMMAD AIMAN BIN ABD AZIS

NPM      : 41155045160075

COURSE: TEKNIK ELEKTRO

JURUSAN TEKNIK ELEKTRO

FAKULTAS TEKNIK

UNVERSITAS LANGLANGBUANA

BANDUNG

DISEMBER 2019

IR REMOTE CONTROL interface with 8051 based on STANDARD PROTOCOL NEC

A. Data path and Control path

Picture 1: Dataflow for IR_tb

Picture 2: Dataflow for IR

Picture 3: Basic Design for Datapath and Controlpath

The datapath is where data moves from place to place.

• Computation happens in the datapath

• No decisions are made here.

• Things you should ﬁnd in a datapath

• Muxes

• Registers

• ALUs

• Wide busses (34 bits  for data.  17 bits for instructions)

• These components are physically large

• In a real machine, their spatial relationship are important.

Control is where decisions are made

• Things you will there

• State machines

• Random lots of complex logic

• Little state (maybe just a single register)

• Spatial relationships are harder to reason about or exploit.

B. ModelSim verilog coding

At line 4

module IR (clk,rst_n,IR,rc ,led_cs,led_db,cs,counter,counter2,data_cnt,led2,get_data);

-The coding of IR module start with input and output desired variable.

- For each input will be declare as wire in module while reg in testbench respectively.

- For each output will be declare as reg in module while wire in testbench respectively.

At line 56

always @ (posedge clk) //triggers the block on the positive (rising) edge of a clock signal

if(!rst_n) //(rst_n = 0 is true, rst_n = 1 is false)

begin

irda_reg0 <= 1'b0;//initialize bit register

irda_reg1 <= 1'b0;//initialize bit register

irda_reg2 <= 1'b0;//initialize bit register

end

else

begin

led_cs = 4'b0000;

irda_reg0 <= IR;

irda_reg1 <= irda_reg0;

irda_reg2 <= irda_reg1;

end

- irda_reg is 3 bit shift register

- irda_reg will non-blocking assignment which is to describe sequential logic

- irda_reg execute in parallel for each line

At line 62

assign irda_chang = (irda_pos_pulse|irda_neg_pulse); // This line is to detect when IR have any change in logic value

assign irda_neg_pulse = (irda_reg2 & (~irda_reg1)); //This line is to detect when IR have change from 1 to 0 in logic value

assign irda_pos_pulse = ((~irda_reg2) & irda_reg1); // This line is to detect when IR have change from 0 to 1 in logic value

At line 75

always @ (posedge clk)

if (!rst_n)

begin

counter <= 9'b0;

end

else if (irda_chang)  //When irda_chang is true, counter will turn to 0

begin

counter <= 9'b0;

end

else if (counter == 9'b111111111)  // When counter = 9'b111111111, counter will turn to 0

begin

counter <= 9'b0;

end

else//Other than that, counter will up by 1 bit per 1 cyle clock

counter <= counter + 9'b1;

//----------------------------------------------------------------------------

always @ (posedge clk)

if (!rst_n)

begin

counter2 <= 10'b0;

end

else if (irda_chang)

begin

counter2 <= 10'b0;

end

else if (counter == 9'b111111111)//Only when counter  count to 511 times, the counter2 will goes                       up by 1 bit.

Begin

counter2 <= counter2 +10'b1;

end

At line 113

assign check_9ms = ((868 < counter2) & (counter2 < 898));

//878 = counter2 will assign check_9ms as true

assign check_4ms = ((429 < counter2) & (counter2 < 449));

//439 = counter2 will assign check_4ms as true

assign low  = ((26 < counter2) & (counter2 < 56));

//35 = counter2 will assign low as true

assign high = ((154 < counter2) & (counter2 < 174));

//164 = counter2 will assign high as true

At line 178

else if (cs == DATA_STATE) //If cs is DATA_STATE then it is true

if (irda_pos_pulse)  // If irda_pos_pulse = 1 then it is true

begin

if (low)  // low is true

begin

get_data <= get_data << 1; //left shift register

get_data[0] <= 1'b0; // 1’b0 will shift into lsb

data_cnt <= data_cnt + 1'b1; // data_cnt goes up by 1-bit

end

else if (high) // high is true

begin

get_data <= get_data << 1; //left shift register

get_data[0] <= 1'b1; // 1’b0 will shift into lsb

data_cnt <= data_cnt + 1'b1; // data_cnt goes up by 1-bit

end

end

else if (irda_neg_pulse)  // If irda_neg_pulse = 1 then it is true

begin

if (!low)  //if low = 0,then it is true

error_flag<=1'b1;

end

At line 203

else if ((data_cnt ==6'd32) & irda_reg1) // If data_cnt decimal value is 32 and irda_reg1 binary value is 1, then it is true

begin

led1 <= get_data[7:0];  // Command(Logical inverse)

led2 <= get_data[15:8]; // Command that will determine output led_db value

led3 <= get_data[23:16];// Address(Logical inverse)

led4 <= get_data[31:24];// Address

end

In file IR_tb.v

- Reg and wire will be shown in simulation wave

- DUT means Device Under Test

- In DUT is for sensitive list, could be seen in IR.v which is always(*)

- Clock cyle is 20ps and reset start  as 0 for 10ps and 1 for the rest of simulation

- rc value must be force input first to give an information which IR signal will we use, for example rc = 4’h0 is for 0 in seven-segment

C. Simulation using Model Sim for input remote switch from 0 until 9

RESULTS FOR INFRARED SIGNAL OF REMOTE CONTROL

Figure 1: rc = 4’d0, led2 = 8’h68, led_db = 8’hc0

Figure 2: rc = 4’h1, led2 = 8’h30, led_db = 8’hf9

Figure 3: rc = 4’h2, led2 = 8’h18, led_db = 8’ha4

Figure 4: rc = 4’h3, led2 = 8’h7a, led_db = 8’hb0

Figure 5: r4 = 4’h4, led2 = 8’h10, led_db = 8’h99

Figure 6: rc = 4’h5, led2 = 8’h38, led_db = 8’h92

Figure 7: rc = 4’h6, led2 = 8’h5a, led_db = 8’h82

Figure 8: rc7 = 4’h7, led2 = 8’h42, led_db = 8’hf8

Figure 9: rc8 = 4’h8, led2 = 8’h4a, led_db = 8’h80

Figure 10: rc9 = 1h’1, led2 = 8h’52, led_db = 8h’90